Scheme of Work - Year 8

| Total Lessons | 7 |  | Design & Make – Key Stage 3 |
| --- | --- | --- | --- |
| Lesson Time | 1hr |  | Electronics |

Objectives Pupils will initially produce a fuse tester in order to gain basic skills and then go onto produce a more complex circuit. Pupils will construct a simple two transistor circuit (darlington pair) which gives a very high gain. The circuit will be constructed using strip board ('Vero board') the circuit is so sensitive that bridging the input wires with a drop of water or even a finger tip or whole body will turn the circuit on. The output will initially be an LED but a buzzer could be substituted if an audible output is needed. Pupils will be required to use the circuit as a solution and find a good practical use for it. both circuits should be suitably housed.

Main Key Experiences & Essential Outcomes Children will be taught:- - Electrical safety. - How to use strip board. - Careful and accurate soldering. - What a transistor looks like - identifying base, collector, emmitter. - What a transistor does - very simple - gain of 50 times from first transistor feeding into second transistor times 50 again =2 500. # Children will be able to:- - Insert components in the correct positions by referring to a drawing. - Make careful good quality solder joints which do not bridge between tracks. - Complete construction of a circuit and then house it in a suitable container to protect it from damage. - Make use of a suitable input sensor. - Test their circuits. # Children will begin to understand:- - Simply, how their circuit works. - That transistors must be inserted correctly and the names of the leads. - Different output devices can be used depending on the situation. - The output sensor will essentially be two conductors that are bridged by a poor conductor. - The need to protect their circuits by a case or housing.

Programmes of Study 1a, 1b, 2a, 3b, 3c, 3d, 3e, 3f, 3g, 3h, 3i, 3j, 3l, 4c, 4d, 4i, 4j, 4k, 6a, 6c, 6d, 6e, 6f, 6g, 8c, 9a, 9c, 10a, 10b, 10c

| Assessment Possible levels: 4-6 - Planning - step by step. - Recording of changes made. - Accuracy during all stages of making - in particular - joints and soldering. - Applying knowledge and understanding - Working with materials. - Evaluating. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Differentiation Worksheets provided In the main by outcome (following instructions, accuracy and finish) according to the groups ability. Additional research can be introduced for extension activities. |  |  |  |  |  |
| RESOURCES |  |  |  |  |  |
| Equipment Tenon saw Bench hook Coping saw Rasps Files Glass paper | Soldering iron Wire cutters Pliers Drill bits Pillar drill Hammer |  | Materials Wood housing MDF Styrene Transistors LEDs | Resistors Capacitors Vero board Various other electronic components. |  |
| Literacy Word search for homework. |  |  |  |  |  |
| Literacy |  | Oral, reading, writing. |  |  |  |
| Numeracy |  | Measuring, marking out |  |  |  |
| ICT opportunities |  | Work sheets |  |  |  |
| Personal qualities |  | Recapping |  |  |  |
| Interpersonal skills |  | Working with each other. |  |  |  |
| Links to other subjects |  | Science, Art, Maths, English |  |  |  |

| The programme |  |
| --- | --- |
| Lesson | Details |
| One | - Introduce pupils to project: Discuss the effects on our modern day life. Life without electricity and especially electronics. - Issue worksheet - touch switch - Explain briefly the function of the circuit - fill in box for possible uses. - Add colour to enhance the sheet and aid in construction. Homework: List 10 electronic / electrical items in the home. State what they do. Briefly state how people managed before electricity. |
| Two | Review Homework - Issue component kits for touch switch. - Demonstrate soldering - use 5 seconds heat and 3 seconds solder technique. - Safety with soldering irons and how to deal with burns. - Use giant models to explain how to place components. - Start practical - students to check each component position with teacher before soldering. |
| Three | - Complete and test circuit. Homework: Electronics word search. |
| Four | Review Homework - The resistor colour code. - Produce coloured diagrams and examples. - Brainstorm ideas for packaging electronic circuits. Using found items and commercial tubes and sheet materials. |
| Five | - Set the design problem - A manufacturer has several thousand of these circuits surplus to needs. Package these into a useful device to turn loss into profit! - Produce a bubble chart to generate ideas such as rain alarm, touch activated door bell, water level in bath, making tea alarm for the blind etc. - Questions to ask before designing product. How big is the battery? Where is the buzzer or LED to be located? How will the circuit be fitted inside the container? Homework: Learn resistor code for test. Collect suitable materials for making containers... |

| Six | Review Homework - Practical demonstration of techniques that can be used to manufacture containers. - tubes, vacuum forming of boxes, engraving machine to cut perfect circles and even complete boxes. wooden tubes and boxes. - Begin design work Homework: Annotate ideas to show simple constructional detail. |
| --- | --- |

| Seven | Review Homework - Assemble - Evaluation - End of project test. |
| --- | --- |

Evaluation

Notes