Scheme of Work - Year 9

| Total Lessons | 5 |  | Design & Make – Key Stage 3 |
| --- | --- | --- | --- |
| Lesson Time | 2hr |  | Metalwork – G-Cramp |

Objectives This project is designed to give an introduction to bench work skills working in mild steel. Pupils will additionally learn how to use the centre lathe to turn a shoulder. Children will work individually and produce the first part by reference to a drawing; the second part - the handle they will design themselves and then manufacture and attach to the cramp, testing to see how effectively it operates. Research into screw threads is also a key part of the work. Product disassembly also features as an important skill with an opportunity for detailed annotation. Looking at exemplars for clamps and uses for screw threads is used as a useful introduction.

Main Key Experiences & Essential Outcomes # Children will be taught :- - To mark out their metal from a drawing. - To work safely with drilling machines. - To cut and file their work effectively. - To cut both internal and external threads. - To work safely with a centre lathe. - To use a jig to bend the cramp. - To rivet. # Children will be able to:- - Deburr their work. - Use layout blue or permanent marker. - Mark parallel lines using odd leg dividers/ surface plate and gauge. - Measure using a steel rule and try square. - Mark metal using a scriber. - Mark curved lines using dividers. - Use a centre punch before drilling. - Drill safely using a clamp or machine vice. - Cut metal using a hacksaw. - File accurately to a line, and finish by draw filing. - Cut an external thread using a die and die holder. - Cut an internal thread using a tap and tap holder. - Turn a shoulder accurately using a centre lathe. - Remove rough edges, clean up and finish their work by painting, lacquering or plastic dip coating. - Attach the washer by riveting. - When manufacturing parts which have to fit together.

| # Children will begin to understand:- - The importance of safety in the various areas of the workshop. - The importance of planning their work and the need to manufacture in the correct sequence. - How to anneal steel by heating and slow cooling. - The need to ensure the handle is both comfortable to use and has sufficient mechanical advantage to be effective, ergonomics. - The need for cutting paste when cutting threads. - That there are three types of tap - taper, second or intermediate and plug; when they should be used. - The cause of drunken threads and how to avoid them. - The basics of turning metal on a centre lathe. - The need for accurate measuring. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Programmes of Study 1a, 1b, 1c 2a 3b, 3g 4a, 4b, 4c, 4f, 4g, 4h, 4j 5a, 5b, 5c, 5d 9a, 9b |  | Assessment Possible levels: 3-5 - Planning - Applying knowledge and understanding - Working with materials - Evaluating |  |  |
| Differentiation Worksheets provided. In the main by outcome (following instructions, accuracy and finish). Design for handles gives scope for extension work. Jigs to aid marking out are available. Bending jigs. Additional research can be introduced for extension activities. |  |  |  |  |
| RESOURCES |  |  |  |  |
| Equipment Exemplar clamps Exemplar screw threads Surface plate and gauge Centre lathe Pillar drills Drill bits-5mm Soft jaws Bending jig | Hand files 6mm taps and dies Centre punch Scribe Permanent markers – blue Ball pein hammers Hacksaws Lead block |  | Materials BDMS bar 10 x 3 BDMS rod – dia.6mm Cutting paste Washers Brazing rod Flux Oil Lacquer |  |
| Literacy Terminology – deburr, odd-legs, de-grease, tap and die, internal and external threads, lubricate, lubricant, jig, drunken thread. Word search. Product disassembly sheets allow opportunities for developing annotation skills. |  |  |  |  |

| The programme |  |
| --- | --- |
| Lesson | Details |
| One | Introduction – G-cramp Examplars – a range of clamps. Discuss action and uses. Present project using engineering drawing (in booklet). Recap mild steel and its properties – BDMS (bright drawn mild steel); mention alternative forms. Demonstrate marking out: - Deburr - Degrease - Stamp initials on bar. - Blue using marker pen; mention layout blue. - Centre line using odd-legs and/or surface plate and gauge - Use of try square, scriber, steel rule. Emphasise accuracy and how to correct errors. - Centre punch and use of dividers to mark out radius - Drilling for tapping. Allow 20minutes for this first part. Practical activity – children to complete marking out and drilling hole for tapping. Ends to be radiused with hand file. Homework: Identify Marking Out Tools – Worksheet (Opportunity for literacy development) |
| Two | Review Homework Demonstrate - thread cutting using tap and die. Need for lubricant (cutting paste). - Centre lathe to cut shoulder – preset - Forming washer using a jig/lead block Practical session. Pupils work on a range of bench work activities, while teacher supervises lathe work. |
| Three | Demonstrate - forming bar into G-cramp shape using jig if available - riveting washer onto treaded rod Practical session. Homework: product disassembly work using empty Pritt stick. (warn about need for this at the beginning) |

| Four and five | Review Homework Final completion and assembly of G-cramp. Design and attach handle - either Tommy-bar or wings. Discuss and look at options for finishing techniques, especially blue and oil or polish and lacquer. Problem with dip coating. Test and evaluate. End of project test Homework: prepare for test; complete process sheets (using worksheet) |
| --- | --- |

Evaluation

Notes